Here is the code for the start of the first operation as shown in the picture:

(ROTARY1 )

M5

M9

T4

S18000 M3

G54

G0 A180.

G0 X-0.262 Y25.

G43 Z6.995 H4

A180.

X-0.138 Z3.684

G1 Z3.677 F833.

Z1.684

X-0.128 Z1.631 F1200.

X-0.098 Z1.583

X-0.052 Z1.549

X0. Z1.536

G93 Z1.535 A173.118 F6453.8939

A-6.875 F248.8696

Z1.533 A-176.102 F264.5653

Z1.532 A-176.33 F193765.3035

Y24.999 Z1.534 A-179.981 F12032.2945

Z1.535 A-180.436 F93164.3823

Z1.536 A-180.891 F92504.8575

Y24.994 A-186.882 F7245.9606

So, if I had before this:

(LEADIN Y3 Z2)

Then the program will need to:

- Look forward from this point, searching for the first instance of Y. Until it finds it, there may be Z values (not in this instance), which will need +2 added to them.

- Finds the first Y value at 25, remembers this number.

- Next line down is Z6.995, because we haven’t moved on Y yet, this still has +2 added to it, becoming Z8.995. Same with multiple more instances of Z on next lines.

- Finds Y24.999, difference of just 0.001, so the addition becomes 2*(2.999/3)=1.999, keeping to 3 decimal places. A difference of 0.001 however is obviously unimportant, so no need to round, just discard any additional decimal places.

Skipping forward a little in the code, one line is:

Y24.739 Z1.588 A-493.523 F8444.98

This will then become:

Y24.739 Z3.414 A-493.523 F8444.98

Because it has moved 25-24.739=0.261. Difference in Z then is 2*(2.739/3)=1.826. Added to the first value of Z1.588 becomes Z3.414

An example for the lead out, except now we are looking at the end of the operation:

Y11.039 Z3.659 A-17030.485 F3418.9517

Y11.036 A-17035.931 F3419.6796

Y11.033 Z3.658 A-17041.377 F3420.5666

Y11.03 A-17046.822 F3421.1605

Y11.027 Z3.657 A-17052.268 F3421.511

Y11.023 A-17057.714 F3421.728

Y11.02 Z3.655 A-17063.938 F2992.0261

Y11.017 Z3.654 A-17069.384 F3424.1623

Y11.014 A-17074.831 F3424.1484

Y11.011 Z3.653 A-17080.277 F3424.0609

Y11.008 Z3.652 A-17085.723 F3424.013

Y11.005 Z3.653 A-17090.392 F3995.9951

Y11.002 Z3.651 A-17095.838 F3421.9218

Z3.649 A-17097.278 F12723.5212

Y11.001 Z3.648 A-17097.566 F64487.1743

Y11. Z3.65 A-17099.87 F8068.8124

Z3.651 A-17100.158 F63286.1954

A-17279.903 F104.7644

Z3.652 A-17455.268 F107.354

A-17460. F3978.2772

G94 X0.053 Z3.664 F1200.

X0.1 Z3.697

X0.132 Z3.746

X0.142 Z3.799

G0

Z5.799

X0.171 Z6.998

(LEADOUT Y2 Z2)

The program now needs to do the same thing, but running upwards through the lines:

- Look backwards from this point, adding 2 to any values of Z until it sees the first Y, which is Y11 (a decimal point is added in code, but with no number after it, this can be kept or discarded, unimportant). +2 needs to be added to the Z on that line.

- Y is now increasing as you go up the lines, and values of Z added need to. The top line in this section of g-code above is:

Y11.039 Z3.659 A-17030.485 F3418.9517

Which is modified to become

Y11.039 Z5.62 A-17030.485 F3418.9517